# Drill

### After landing

- exit to taxiway

- flaps UP

- aircraft lights

  - beacon light ON

  - landing light OFF

  - taxi light ON

  - nav light ON

  - strobe light OFF

- transponder STANDBY

- trim SET to TAKE OFF

- radio ARCHER GROUND ALPHA FOXTROT ROMEO REQUEST TAXI TO THE EASTERN APRON

- tower ALPHA FOXTROT ROMEO CLEARED FOR TAXI TO THE EASTERN APRON

##### Engine shut off

- parking brake SET

- electrical equipment OFF

  - all lights except navigation beacon OFF

- avionics master OFF

- mixture IDLE CUT OFF

- master switch OFF

- magnetos switch OFF

- control lock INSTALL

- fuel selector valve LEFT or RIGHT to prevent cross feed

- pitot tube cover