# Drill

### Circuits

##### Right circuits in VH-AFR, runway 28R

- take-off

- set full power

- 55KIAS rotate

  - apply right rudder to keep balance

- 300ft after take-off check

  - flaps up

  - power is full

  - > 2300rpm

  - landing light [can leave on for circuits]

  - achieve 74KIAS (Vy)

- at 500ft turn onto crosswind

  - lookout [for right turn]: clear left, clear centre, clear above, clear right

  - roll out when heading bug at 90 degrees

  - maximum AoB 15 degrees [in climb]

  - adjust attitude for 74KIAS (Vy)

  - level off at 1000ft

- at 1nm from runway turn onto downwind

  - lookout [for right turn]: clear left, clear centre, clear above, clear right

  - AoB 30 degrees

  - rollout to straight & level when heading bug at 180 degrees

  - radio ALPHA FOXTROT ROMEO DOWNWIND FOR TOUCH AND GO

  - BUMFISH check

    - Brakes

    - Under-carriage

    - Mixture full rich

    - Fuel on both and sufficient to go-around

    - Instruments in green

    - Switches, landing light ON

    - Hatches and Harnesses secure

  - hold the heading with visual reference point

  - workflow: Height, Speed, Heading, Spacing

  - abeam runway threshold reduce power to 2000rpm

  - maintain 1000ft

  - at below 110KIAS take [first stage] 10 flap

  - trim for flap

- at 45 degrees from runway threshold turn onto base

  - lookout [for right turn]: clear left, clear centre, clear above, clear right

  - roll out when heading bug at 270 degrees

  - power back to 1500rpm

  - maximum AoB 30 degrees [descent]

  - at below 85KIAS take [second stage] 20 flap

  - trim for flap

  - half-sky/half-ground attitude

  - aim for 75KIAS in approach descent

- turn onto final

  - aim for 75KIAS in approach

  - tower ALPHA FOXTROT ROMEO RUNWAY TWO EIGHT RIGHT CLEAR TOUCH AND GO

  - radio TWO EIGHT RIGHT CLEAR TOUCH AND GO ALPHA FOXTROT ROMEO

  - aimpoint (runway numbers), aspect, airspeed (65)

  - take [full] 30 flap

  - trim for flap

  - aim for 65KIAS

- land

  - at just before threshold markers, power to idle

  - flare: straight and level attitude, look to the distant runway

  - hold off with nose up, horizon at dash cowling

- after landing

  - flaps up