# Drill

### Engine Start

##### Cessna 172S

- checks

  - pre-flight inspection COMPLETE

  - passenger briefing COMPLETE

  - seats and seat belts ADJUST and LOCK. Ensure inertia reel locking.

  - brakes TEST and SET

  - circuit breakers CHECK IN

  - electrical equipment OFF

  - avionics master switch OFF

  - fuel selector valve BOTH

  - fuel shutoff valve ON (push full in)

  - avionics circuit breakers CHECK IN

- prime engine with fuel

  - throttle to 1/2

  - master switch ON

  - beacon light ON

  - fuel pump ON

  - mixture full-rich until stable fuel flow indicated (3 to 5 seconds)

  - mixture full-lean

  - fuel pump OFF

- engine start

  - mixture full-lean

  - throttle to 1/4

  - hand on mixture

  - “CLEAR PROP”

  - crank engine to start

  - mixture to full-rich

- check

  - oil pressure

  - navigation beacon ON

  - avionics master switch ON

  - radios ON

  - flaps UP